DNC-700H Color High Resolution Day & Night Camera - 700 TV Line - 0.000002 Lux in Sens-Up Mode - On Screen Display - Dead Pixel Compensation

*Above specifications and appearance of product subject to change without notification Total Pixels; 811H x 508V (NTSC), 795H x 596V (PAL). Effective Pixels; 768H x 494V (NTSC), 752H x 582V (PAL). Sensor; 1/3” High Resolution CCD. Scanning System; 2:1 Interlace. Frequency; 15.734KHz x 59.94Hz (NTSC), 15.625KHz x 50.00KHz (PAL). Resolution; 700 TV Lines. Video Out; 1.0Vp-p composite signal (75 ohm). S/N (Y Signal; 52dB or more (AGC off). Min. Illumination; 0.01 Lux. Gain Control; Off / Low / Middle / High. White Balance; ATW1 / ATW2 / AWC / >> Set / Anti. CRL / Manual. Electronic Shutter Speed; NTSC: 1/60sec ~1/100,000sec, PAL: 1/50sec ~1/100,000sec. BLC; Off, BLC, HLC Selectable. 2D-DNR; On (0~15) /Off Selectable. Day & Night Selection; Auto/Color/B&W/EXT. D-WDR; Off / low level (0~15) / high level (0~15). Dead Pixel (DPC); Selectable. IR Smart; On/Off Selectable (IR Gain Level Adjustable 0~255). Motion Detection; Off/On (4 Areas Selectable). Privacy Masking; Off/On (8 Masks Selectable). Lens Shading (LSC); On/Off Selectable (Level Adjustable 0~255). Sharpness; On/Off (Level Adjustable 1~31). OSD; Available. Lens Mount; C/CS Mount. Operating Temperature; -10 ~ +50°C / 0~80% RH. Power Supply; DC 12V / AC 24V 150mA. Dimensions; 65W x 52H x 108D mm. Weight; Appx. 350g. *Lens not included