KPC-WDR470CAV10 ## Vandal Proof Wide Dynamic Range Dome Camera - 1/3” Sony Double Denisty CCD - 480 TV Lines - 2.8~11mm Varifocal Lens - Digital Slow Shutter - Mechanical Day & Night - Dual Voltage - 0.004 Lux (with AGC Off, DSS 40x)

*Above specifications and design subject to change without notification. Signal Format; NTSC. Image Sensor; 1/3” Sony Vertical Double Density CCD. Scanning System; 2:1 Interlaced. Horizontal Resolution; 480 TV Lines. S/N Ratio; More than 50dB (AGC, DSS Off). Minimum Illumination; Color: 0.004 Lux, B&W: 0.0001 Lux / AGC TYP, DSS 40x. Electronic Shutter; 1/60~1/100,000 Sec. BLC; Off/On (Level Adjustment, Area Selection). WDR; Off/On (Level Adjustment). AGC; Min/TYP/Max. Gamma; r=0.45 / 1 / Active Mode 1 / Active Mode 2. Day & Night; Off / On (Video, CDS Sensor), IR Remove. White Balance; ATW / Push Lock / User (R Gain, B Gain). Digital Slow Shutter; Off / On (2x, 4x, 6x, 8x, 10x, 20x, 40x, 80x, 160x Field). Manual Shutter; 8 Steps (1/60, 1/100, 1/250, 1/500, 1/1k, 1/2k, 1/4k, 1/10k). Image Effect; B&W, Nega/Posi, H/V Mirror, Sharpness, Freeze. Color Adjustment; R-Y Gain, B-Y Gain, R-Y Hue, B-Y Hue. Motion Detection; Off / On ( Level Control, Area Setting). Baud Rate; 2400 / 4800 / 9600 / 19200 bps. Character Generator; 10 Alphanumeric Character. Flickerless; Off / On (WDR On). Privacy Zone; 8 Point. Sync System; Internal. Power Supply; DC 12V ±10% / AC 24V ±10%. Lens; 2.8~11mm Varifocal Auto Iris. Operating Temperature; -10°~+50°C. Storage Temperature; -20~+60°C.