LENS-M3-8 3~8MM VARIFOCAL MANUAL IRIS LENS - Variable Focal Length (3 ~ 8mm) - Manual Iris - Standard CS Mount

Focal Length; 3~8mm. Max. Aperture Ratio; 1:1.4. Max. Image Format Size; 4.8 x 3.6 (Ø 6) mm. Operation; Iris; F 1.4 ~ close. Focus; Manual. Zoom; Manual. Angle of View; Diagonal; 1/3”. Horizontal; 1/3”. Vertical; 1/3”. Optical Distortion; 1/3”. Flange Back Focal Length; 12.5mm. Back Focal Length; 8.70 ~ 14.37mm. Mount; CS mount. Operation Temperature; -10~50°C. Dimensions (inch); 1.8 x 2.4. Weight; 0.2 lbs. *Above specifications and design subject to change without notification